GMX1200S

The K-tronic holder/charger unit GMX1200S fitting MOTOROLA GP 900/GP1200 hand radio sets is a holding device with an integrated battery charger, which switches off automatically, when the battery is fully charged. The unique design allows the usage of all standard batteries (e.g. NTN7143 - 1300mAh and NTN7144A - 1500mAh).

Criteria for cutting off the charging process are voltage differences (V), rising battery temperatures of over 50°C or exceeding the time limit. The electronic charging circuit cuts in automatically, should the battery voltage drop below a certain limit whilst the radio set is in operation. The availability of at least 60 - 70% of the battery capacity is thereby ensured at any time. In addition, switching the lever at the side of the casing transfers the speaker and microphone functions to the external hand microphone and switches the radio aerial to the BNC plug at the bottom of the casing. The radio set is ready for transmission by pressing the PTT-button on the hand microphone.

Technical Data

Working voltage 10 to 28 Volt / DC

Current uptake without battery 0 mA

Fuse USA F2A

Current uptake with Battery

at 13.3 Volt during charging input current: approx. 800 mA

* charging current: approx. 900 mA

at 24 Volt during charging input current: approx. 450 mA

* charging current: approx.900 mA

Dimensions (W/H/D) 87x170x 57mm

Colour RAL 9005, structured